June 2012

| Americas Bosch Security Systems, Inc. 130 Perinton Parkway Fairport, New York, 14450, USA Phone: + 1 800 289 0096 Fax: +1 585 223 9180 security.sales@us.bosch.com www.boschsecurity.us | Europe, Middle East, Africa Bosch Security Systems B.V. P.O. Box 80002 5600 JB Eindhoven, The Netherlands Phone: + 31 40 2577 284 Fax: +31 40 2577 330 emea.securitysystems@bosch.com www.boschsecurity.com | Asia-Pacific Robert Bosch (SEA) Pte Ltd, Security Systems 11 Bishan Street 21 Singapore 573943 Phone: +65 6571 2600 Fax: +65 6571 2698 apr.securitysystems@bosch.com www.boschsecurity.com |
| --- | --- | --- |

Product Guide Specification

Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format, based on MasterFormat 2004 and The Project Resource Manual—CSI Manual of Practice. The Manufacturer is responsible for technical accuracy.

The section must be carefully reviewed and edited by the Architect or Engineer to meet the requirements of the project and local building code.  Words and sentences within brackets [ ] are choices to include or exclude a particular item or statement.  Coordinate this section with other specification sections and the Drawings.  Delete all “Specifier Notes” after editing this section.

SECTION 28 23 29

VIDEO SURVEILLANCE REMOTE DEVICES AND SENSORS

BOSCH AUTODOME JUNIOR HD FIXED CAMERA

- – GENERAL

  - SUMMARY

    - Section Includes

      - Security Lighting

    - Related Sections

      - Section [28 23 13 – Video Surveillance Control and Management Systems].

      - Section [28 23 16 – Video Surveillance Monitoring and Supervisory Interfaces].

      - Section [28 23 19 – Digital Video Recorders and Analog Recording Devices].

      - Section [28 23 23 – Video Surveillance Systems Infrastructure].

**********Specifier’s note: Include those standards referenced elsewhere in this SECTION.

  - REFERENCES

    - Canadian Standards Association (CSA)

      - Complies with CSA standards.

    - European Norm

      - Complies with CE Electromagnetic Compatibility regulations

      - Complies with CE Product Safety regulations

      - Complies with EN Product Safety standards

      - Complies with EN 50130-4:1995 + A1:1998 + A2:2003 - Alarm Systems - Electromagnetic Compatibility - Product Family Standard: Immunity Requirements for Components Of Fire, Intruder And Social Alarm Systems.

Note: Requires the use of the one of the following power supply units: 
VG4-A-PSU0, VG4-A-PSU1, or VG4-A-PSU2.

    - Federal Communications Commission (FCC) (www.fcc.gov)

      - Complies with FCC Part 15.

    - Interference-Causing Equipment Standards

      - Complies with ICES-003 regulations.

    - International Electrotechnical Commission (IEC)

      - Complies with IEC Product Safety standards

    - Underwriters Laboratories, Inc. (UL) (www.ul.com)

      - Complies with UL standards.

  - DEFINITIONS

    - Privacy Masking: The ability to mask out a specific area to prevent it from being viewed in order to comply with privacy laws and particular site requirements.

    - SensUp (sensitivity up):  Increases camera sensitivity by increasing the integration time on the CCD (lowering shutter time from 1/50 to 1/5 s). This is accomplished by integrating the signal from a number of consecutive video fields to reduce signal noise.

    - Automatic Gain Control (AGC):  a process by which gain is automatically adjusted as a function of input or other specified parameter.

    - Sensitivity: refers to the minimum level of light the CCD chip needs to generate an acceptable video picture, and is measured in lux.

  - SYSTEM DESCRIPTION

    - Section Includes

      - Video Surveillance Remote Devices

    - Performance Requirements

      - The unit shall be a compact, cost effective HD fixed camera unit designed for discrete video surveillance applications in indoor environments.

      - The camera shall offer remote commissioning to set the position and the zoom level of the camera mechanism from a head-end unit.

      - The camera shall provide full HD 1080p30, 720p60, and 720p30 resolutions.

      - The camera shall provide direct network connection using H.264 and JPEG compression and bandwidth throttling to efficiently manage bandwidth and storage requirements while delivering outstanding image quality.

      - The camera shall offer embedded Intelligent Video Analysis (IVA) that eliminates dedicated PCs and associated software maintenance.

      - The camera shall generate multiple simultaneous video streams:

        - High Definition Streams:

          - [One (1) H.264 1080p30 or 720p60 stream plus one (1) I-frame stream plus one (1) M-JPEG simultaneously]

          - [One (1) independently configurable H.264 720p30 stream plus one (1) independently configurable H.264 Baseline Plus (BP+) or H.264 Main Profile Standard Definition (MP SD) plus one (1) I-frame plus one (1) M-JPEG video stream simultaneously]

        - Standard Definition Streams:

          - One (1) H.264 BP+ or H.264 MP SD stream plus one (1) H.264 BP+ or H.264 MP SD stream plus one (1) I-frame stream plus one (1) M-JPEG stream simultaneously

      - The camera shall offer Power over Ethernet+ (IEEE 802.3at, class 4) for indoor applications.

      - The camera shall be a high performance 1/2.5-in. progressive scan day/night CMOS sensor with up to 1920 x 1080 resolution.

      - The camera shall offer a wide dynamic range of 50 dB.

      - The camera shall have an autofocus lens with 160x zoom (10x optical/16x digital).

      - The dome housing shall be a vandal resistant, durable, rugged design with an acrylic bubble.

  - SUBMITTALS

    - Submit under provisions of Section [01 33 00.]

    - Product Data:

      - Manufacturer’s data, user and installation manuals for all equipment and software programs including computer equipment and other equipment required for complete video management system.

    - Shop Drawings; include

      - System device locations on architectural floor plans.

      - Full Schematic of system, including wiring information for all devices.

    - Closeout Submittals

      - User manual.

      - Parts list.

      - System device locations on architectural floor plans.

      - Wiring and connection diagram.

      - Maintenance requirements.

  - QUALITY ASSURANCE

    - Manufacturer:

      - Minimum of [10] years experience in manufacture and design Video Surveillance Devices.

    - Video Surveillance System:

      - Listed by CSA.

      - Certified compliant to FCC and CE for the required loads.  Test methods are in accordance with Industry Canada and the IEC.  Provide evidence of compliance upon request.

    - Installer:

      - Minimum of [5] years experience installing Video Surveillance System.

  - DELIVERY, STORAGE AND HANDLING

    - Comply with requirements of Section 01 60 00.

    - Deliver materials in manufacture’s original, unopened, undamaged containers; and unharmed original identification labels.

    - Protect store materials from environmental and temperature conditions following manufacturer’s instructions.

    - Handle and operate products and systems according to manufacturer’s instructions.

    - Bosch provides off-the-shelf availability for our top selling products and same-day or 24-hour shipping.

  - WARRANTY

    - Provide manufacturer’s warranty covering 3 years for replacement and repair of defective equipment.

  - MAINTENANCE

    - Make ordering of new equipment for expansions, replacements, and spare parts available to dealers and end users.

    - Provide factory direct technical support from 8:00 a.m. to 8:00 p.m. via phone and e-mail.

- – PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer:

[Bosch Security Systems, Inc.

130 Perinton Parkway

Fairport, New York, 1450, USA

Phone: + 1 800 289 0096

Fax: + 1 585 223 9180

security.sales@us.bosch.com

www.boschsecurity.us]

[Bosch Security Systems B.V.

P.O. Box 80002

5600 JB Eindhoven, The Netherlands

Phone: + 31 40 2577 284

Fax: +31 40 2577 330

emea.securitysystems@bosch.com

www.boschsecurity.com]

[Asia-Pacific

Robert Bosch (SEA) Pte Ltd, Security Systems

11 Bishan Street 21

Singapore 573943

Phone: +65 6571 2600

Fax: +65 6571 2698

apr.securitysystems@bosch.com

www.boschsecurity.com]

    - Substitutions: [Not permitted.] [Under provisions of Division 1.]

      - [All proposed substitutions must be approved by the Architect or Engineer professional.]

      - [Proposed substitutions must provide a line-by-line compliance documentation.]

  - BOSCH AUTODOME JUNIOR HD FIXED CAMERA

    - General Characteristics:

      - The compact fixed camera shall offer remote commissioning to set the position and the zoom level of the camera mechanism from a head-end unit.

      - The compact fixed camera shall provide a 1/2.5-in. Progressive scan day/night CMOS camera with the following:

        - Up to 1920 x 1080 (NTSC/PAL) resolution.

        - Sensitivity to below 1.0 lux.

      - The compact fixed camera shall offer a wide dynamic range of 50 dB.

      - The compact fixed camera shall provide direct network connection using H.264 and JPEG compression and bandwidth throttling to efficiently manage bandwidth and storage requirements while delivering outstanding image quality.

      - The compact fixed camera shall offer Power over Ethernet+ (IEEE 802.3at class 4) for indoor applications.

      - The compact fixed camera shall provide 15 individual privacy masks with the following features:

        - Shapes shall be defined with four corners.

        - The mask shall be black, gray, or white.

      - The compact fixed camera shall be fully assembled and ready for surface mount applications.

      - The compact fixed camera shall be able to be mounted to a wall, recessed into a ceiling, mounted to a surface, or mounted to a pipe.

      - The compact fixed camera shall support the following languages:

        - Chinese, Simplified

        - Dutch

        - English

        - French

        - German

        - Italian

        - Japanese

        - Polish

        - Portuguese

        - Russian

        - Spanish

    - HD Characteristics

      - The compact fixed camera shall generate full HD 1080p resolution and 720p resolution using H.264 compression.

      - The camera shall generate multiple simultaneous video streams:

        - High Definition Streams:

          - [One (1) H.264 1080p30 or 720p60 stream plus one (1) I-frame stream plus one (1) M-JPEG simultaneously]

          - [One (1) independently configurable H.264 720p30 stream plus one (1) independently configurable H.264 Baseline Plus (BP+) or H.264 Main Profile Standard Definition (MP SD) plus one (1) I-frame plus one (1) M-JPEG video stream simultaneously]

        - Standard Definition Streams:

          - One (1) H.264 BP+ or H.264 MP SD stream plus one (1) H.264 BP+ or H.264 MP SD stream plus one (1) I-frame stream plus one (1) M-JPEG stream simultaneously

    - IP Connectivity

      - The compact fixed camera shall allow full camera control and configuration capabilities over the network.

      - The compact fixed camera shall offer Power over Ethernet+ (IEEE 802.3at class 4) for indoor applications.

      - The compact fixed camera shall be capable of capturing and storing images using H.264 compression at HD 1080p and HD 720p resolutions.

      - The compact fixed camera shall deliver DVD-quality 1080p video, at rates up to 30 images per second, via TCP/IP over Cat5/Cat6 UTP cable.

      - The compact fixed camera shall support iSCSI devices to allow the network-enabled camera to stream video directly to an iSCSI RAID array.

      - The compact fixed camera shall conform to the ONVIF standard.

      - The compact fixed camera shall offer embedded Intelligent Video Analysis (IVA) that eliminates dedicated PCs and associated software maintenance.

    - Intelligent Video Analysis

      - The compact fixed camera shall be capable of processing and analyzing video within the camera itself, with no extra hardware required.

      - The compact fixed camera shall be capable of detecting and sending alarms for abnormal events.

      - The compact fixed camera shall allow users to set up to 10 separate profiles and switch profiles based on a day/night or holiday schedules.

    - Camera:

      - Imager: 1/2.5-inch progressive scan day/night CMOS

      - Effective Pixels: 2.1 Megapixels

      - Lens:

        - 10x optical zoom, f=6.3 to 63 mm

        - 16x digital zoom

      - Focus: Automatic with manual override

      - Iris: Automatic with manual override (F1.8 to F2.5)

      - Field of View:

        - TELE end: 5.8°

        - WIDE end: 53°

      - Automatic Gain Control: Normal, Middle, High, Off adjustable (gain control in Off mode)

      - Synchronization: Internal

      - Aperture correction: On/Off

      - Horizontal Resolution: 1920 pixels

      - View Angle:

        - Horizontal

          - 16:9 aspect: 5.5 to 50.6°

          - 4:3 aspect: 5.2 to 48.7°

        - Vertical

          - 16:9 aspect: 3.1 to 29.3°

          - 4:3 aspect: 3.9 to 37°

        - Diagonal

          - 16:9 aspect: 6.2 to 57°

          - 4:3 aspect: 6.5 to 58.8°

      - Sensitivity (Tinted bubble adds 0.8 f-stop loss)

        - 30 IRE

          - Day Mode

            - SensUP Off: 1.2 lx

            - SensUP On (64x): 0.038 lx

          - Night Mode

            - SensUP Off: 0.06 lx

            - SensUP On (64x): 0.0018 lx

        - 50 IRE

          - Day Mode

            - SensUP Off:  2.0 lx

            - SensUP On (64x): 0.063 lx

          - Night Mode

            - SensUP Off: 0.1 lx

            - SensUP On (64x): 0.0031 lx

      - Signal-to-Noise Ratio: > 50 dB (AGC off)

      - Wide Dynamic Range: 50 dB

      - Backlight Compensation: On/Off

      - White Balance: ATW, Indoor, Outdoor, AWB Hold, Extended ATW, Manual (adjustable R and B signals in Manual mode)

      - Electronic Shutter: 1x to 64x; 1/50 (1/60) to 1/10000

      - Gamma: 0.45, 1 (other settings available)

      - Day/Night: Monochrome, Color, Auto

    - IP Video

      - Standards: H.264 (ISO/IEC 14496-10), M-JPEG, JPEG

      - Streaming: Multiple individually configurable streams in H.264 and JPEG, configurable frame rate and bandwidth:

        - One (1) H.264 1080p30 or 720p60 stream plus one M-JPEG and one I-frame stream simultaneously.

        - One (1) independently configurable H.264 720p30 stream and one (1) independently configurable H.264 Baseline Plus (BP+) or H.264 Main Profile Standard Definition (MP SD)

        - Two (2) independently configurable H.264 streams:

          - One (1) H.264 720p30 stream,

          - One (1) H.264 BP+ or H.264 MP SD stream,

          - plus one (1) M JPEG and one (1) I-frame stream

        - Two (2) independently configurable standard definition H.264 streams:

          - One (1) H.264 BP+ or H.264 MP SD stream,

          - One (1) H.264 BP+ or H.264 MP SD stream,

          - plus one (1) M JPEG and one (1) I-frame stream

      - GOP Structure: IP, IBP, IBBP

      - Data Rate: 9.6 kbps to 6 Mbps

      - Overall IP delay: 240 ms

      - Resolution

        - High Definition:

          - HDTV 1080p: 1920 x 1080

          - HDTV 720p: 1280 x 720

        - Standard Definition (50/60 Hz):

          - 4CIF/D1: 704 x 576/480

          - CIF: 352 x 288/240

      - Frame Rate:

        - 1080p: 1 to 30 ips

        - 720p: 1 to 60 ips or 1 to 30 ips

    - Network

      - Protocols: RTP, Telnet, UDP, TCP, IP, HTTP, HTTPS, FTP, DHCP, IGMP V2/V3, ICMP, ARP, SMTP, SNTP, SNMP, RTSP, 802.1x, iSCSI, DynDNS, UPnP

      - Ethernet: 10-Base T/100 Base-TX, auto-sensing, half/full duplex, RJ45

      - Power over Ethernet+ (Indoor models only): IEEE 802.3at, class 4

    - Electrical

      - Camera Rated Voltage: Indoor Rated Voltage

        - [24 VAC ± 10%, 50/60 Hz]

        - [Power over Ethernet+ (IEEE 802.3at class 4)]

      - Power Draw (typical): 14 W

    - Mechanical

      - Size (Diameter x Height): 153.0 x 133.0 mm (6.02 x 4.61 in.)

      - Bubble Size: 128.0 mm (5.04 in.)

      - Weight: 1.92 kg (4.24 lb)

      - Construction:

        - Housing: Cast aluminum

        - Bubble: Acrylic

      - Color:

        - [White]

        - [Charcoal]

    - Environmental:

      - Operating Temperature: –10°C to 40°C (14°F to 104°F)

      - Storage Temperature: –20°C to 60°C (–4°F to 140°F)

      - Operating Humidity: 0% to 90% relative, non-condensing

    - User Connections:

      - Power (camera):

        - [24 VAC, 50/60 Hz]

        - [RJ-45 100 Base-TX Ethernet Power over Ethernet+ IEEE 802.3at class 4]

      - Control: RJ-45 100 Base-TX Ethernet

      - Video: RJ-45 100 Base-TX Ethernet

      - Alarm Input (2): NC/NO

      - Alarm Output: Open collector NPN, 32 VDC @ 150 mA, maximum

      - Audio: 9 K Ohm typ, 5.5 Vp-p max.

- – EXECUTION

  - EXAMINATION

    - Examine areas to receive devices and notify adverse conditions affecting installation or subsequent operation.

    - Do not begin installation until unacceptable conditions are corrected.

  - PREPARATION

    - Protect devices from damage during construction.

  - INSTALLATION

    - Install devices in accordance with manufacturer’s instruction at locations indicated on the floor drawings plans.

    - Perform installation with qualified service personnel.

    - Install devices in accordance with the National Electrical Code or applicable local codes.

    - Ensure selected location is secure and offers protection from accidental damage.

    - Location must provide reasonable temperature and humidity conditions, free from sources of electrical and electromagnetic interference.

  - FIELD QUALITY CONTROL

    - Test snugness of mounting screws of all installed equipment.

    - Test proper operation of all video system devices.

    - Determine and report all problems to the manufacturer’s customer service department.

  - ADJUSTING

    - Make proper adjustment to video system devices for correct operation in accordance with manufacturer’s instructions.

    - Make any adjustment of camera settings to comply with specific customer’s need.

  - DEMOSTRATION

    - Demonstrate at final inspection that video management system and devices functions properly.

END OF SECTION